Résumé

L’objectif de  cette étude est d’évaluer un procédé d’adsorption-désorption d’un polluant émergent, à savoir l’amoxicilline  présent dans une solution aqueuse synthétique à des faibles concentrations, sur un sous produit de palmier dattier (pédicelle) activé chimiquement,. Une étude paramétrique a été effectuée pour examiner l’influence de certains paramètres opératoires sur la capacité d’adsorption à savoir : le temps de contact, le pH, la dose du charbon, la température et la concentration initiale de l’amoxicilline. L’étude cinétique a montré que l’équilibre d’adsorption est atteint au bout des premières 30 min de contact. L’isotherme d’adsorption de l’amoxicilline a été étudiée en appliquant différents modèles

Les résultats obtenus indiquent que le modèle de Freundlich (R2 = 0,933) décrit convenablement les résultats expérimentaux pour l’amoxicilline. En outre l’étude cinétique de l’adsorption révèle que la vitesse d’adsorption  est régie par le modèle du pseudo second ordre.  La détermination des paramètres thermodynamiques a permis de montrer  que le processus d’adsorption  est non spontané et exothermique.

Les résultats obtenus  pour l’étude de la désorption sont  très satisfaisants.  Des rendements intéressants ont été enregistrés pour la désorption de l’amoxicilline par  une solution aqueuse de NaOH à 5%.

Les perspectives de ce travail de recherche sont nombreuses ; elles consistent à approfondir l’étude par la réalisation d’autres essais qui porteront sur le traitement physique du précurseur ou chimique avec d’autres agents activant.

Abstract

The objective of this study is to evaluate a process of adsorption-desorption of an emergent pollutant, namely the amoxicilline present in a synthetic aqueous solution at weak concentrations, about under product of date palm (pedicel) activated chemically. a parametric study was carried out to examine the influence of certain operational parameters on the capacity of adsorption to knowing:¶  the time of contact, pH, the amount of coal, the temperature and initial concentration of the amoxicilline.¶  The kinetic study showed that the balance of adsorption is reached at the end of the first 25 min contact.¶  The isotherm of adsorption of the amoxicilline was studied by applying various models ¶

The results obtained indicate that the model of Freundlich (R2 = 0,933) suitably described the experimental results for the amoxicilline.¶  Moreover the kinetic study of adsorption reveals that the speed of adsorption is governed by the model of the pseudo second order. ¶The determination of the thermodynamic parameters made it possible to show that the process of adsorption is nonspontaneous and exothermic.¶

Desorption is thus favored in a basic medium with 5% NaOH with a higher output of 87 %.

¶The prospects for this research task are numerous; ¶they consist in looking further into the study by the realization of other tests which will relate to the physical of precursor or chemical treatment with other agents activating ¶