Résumé

Ce mémoire présente l'étude et la mise en œuvre d'une méthode de détection et d'extraction des bâtiments dans le milieu urbain à partir des images multispectrales à très haute résolution. La méthode est basée sur la combinaison de l’information spectrale et spatiale de chaque pixel. Le système développé se compose globalement de deux séquences de traitements. La première permet le rehaussement des bâtiments en se basant sur l’analyse de la similitude spectrale entre les toitures des bâtiments. Dans la seconde phase, une extraction des bâtiments rehaussés est élaborée en faisant appel aux outils de la morphologie mathématique floue. La transformation HMT (Hit or Miss Transformation) floue permet la discrimination des bâtiments par rapport à leur environnement. Des critères d'évaluation ont été développés. Ils sont basés sur le taux d'identification et sur la bonne localisation des bâtiments.

Cette méthode a été appliquée à des images de différentes résolutions spatiales. Les bâtiments ainsi recherchés ont été extraits indépendamment de leur taille, forme, et orientation.

Abstract

This thesis presents the study and the implementation of a method of detection and extraction of buildings in urban areas from multispectral images at very high resolution. The method is based on the combination of the spectral and spatial information of each pixel. The developed system is generally comprised of two sequences of treatment. The first allows the enhancement of buildings based on the analysis of the spectral similarity between the roofs of buildings. In the second phase, an enhanced extraction of buildings is developed by using the tools of fuzzy mathematical morphology. HMT (Hit or Miss Transform) fuzzy allows the discrimination of buildings relative to their environment. Evaluation criteria were developed. They are based on the rate of identification and the proper location of buildings as well.

This method has been applied to images of different spatial resolutions. The well sought after buildings were extracted regardless of their size, shape, and orientation.